ATP QUICK REFERENCE – BRIEFINGS AND COCKPIT PROCEDURES

Based on FAA-S-8081-5D, ATP Airplane Practical Test Standards

Assumes reciprocating multi-engine airplane under 6000 lbs., Vso less than 61 knots IAS.

Pre-taxi and Taxi

- Organize cockpit (charts, checklists, etc.) logically and with attention to CRM issues.

- Complete startup and pre-taxi checklist.

- Use airport diagram during taxi.

- Complete instrument checks during taxi:

  - Compass: free and correct, no bubbles, not leaning, shows a known heading, compass deviation card installed.

  - ASI: correct.

  - AI: leans no more than 5 degrees left or right

  - ALT: within 75 feet of field elevation after set to barometric pressure

  - Rate of turn indicator: shows turn in proper direction

  - Inclinometer: ball slides away from direction of turn

  - DG or HSI: shows turn in proper direction, agrees with magnetic compass

  - VSI: zero or note indication to benchmark

  - Clock: verify set and running

Pre-takeoff briefing

  - Runway (length, width, displaced threshold if applicable)

  - SID/STAR/FMSP/IAP

  - Takeoff performance to include runway length used, based on:

    - Wind

    - Density altitude

    - Weight

    - Temperature

    - Pressure altitude

    - Runway condition and length

    - Obstructions

  - Airspeeds and power settings

    - Airspeed for rotation and climb

    - MAP and RPM for takeoff and initial climb segments

  - Abnormals or emergency prior to or after reaching decision speed (i.e., V1 or Vmc)

  - Emergency return intentions

  - What is expected of the other crewmembers during the takeoff/DP

  - If the first takeoff/departure and approach/landing briefings are satisfactory, the examiner may allow the applicant to brief only the changes, during the remainder of the flight.

- Emergency and abnormal procedures

  - Abort plan

  - Accelerate-stop distance (use performance chart from AFM)

Approach and landing briefing

- Thorough IAP Brief, must include at least:

  - Missed approach procedures

  - FAF

  - Altitude at FAF

  - Initial rate of descent

  - DA/DH/MDA

  - Time to missed approach

  - What is expected of the other crewmembers during the approach/landing.

- Configuration

  - Stabilized airspeed for approach

    - Gust/wind factor (apply ½ the gust factor to approach airspeed)

  - Power settings

  - Flap configuration

- Runway length and condition

- Touchdown point (PTS standard: 500 to 3000 feet beyond threshold)

- Landing distance

- Meteorological conditions’ effect on landing performance

- Brief go-around procedures (pitch up, power up, clean up)