ATP Quick Reference: Formulae

# Formulae used in Performance Calculations

All examples reference Piper PA-30B speeds/weights.

Calculating Va: (maneuvering speed)

Sqrt of (TOW / MGTOW) * published Va (162mph)

Example:

Actual weight: 3400

MGTOW: 3600

Sqrt of 3400/3600 = 0.972  *  162 (published Va) = 157.44

Calculating lead intercepts for DME arcs and any selected course

TAS / 200

Example:

120KTAS / 200 = 0.6

Therefore, initiate a standard rate turn 0.6nm prior to arriving at the arc.

For a 45 degree intercept to the final approach course, 0.3nm
(use GPS for XTRK information.)

For a 30 degree intercept, initiate at 0.2nm.

Calculating Rate of Descent to track glideslope

(Groundspeed / 60) * 300

Example:

105 knots GS

105/60 = 1.75

1.75 * 300 = 525FPM

Note: 300 is used because flying a 3-degree glideslope will cause the aircraft to descend 300 feet per nautical mile (FPNM.)  If a nonstandard glideslope angle is published on the IAP, use that number instead (i.e. if 3.24 degrees, use 324.)

Alternate method (rule of thumb):

Use 10% of groundspeed, and multiply times 5.

Example:

90 knots GS

90 * 5 = 450FPM

This method is quite accurate, provided a standard 3.00 degree glideslope.

Initiating level-off from climb or descent

10% of Rate of Climb/Descent

Example:

If climbing at 500FPM, initiate level-off 50 feet below target altitude.

Calculating bank angles for standard rate turns

10% of TAS / ½ of that number

Example:

120KTAS

12 + 6 = 18 degrees angle of bank

Calculating roll-out from steep (45 degree bank angle) turns

Initiate roll-out at a heading equivalent to ½ the bank angle.

Example:

45 degree bank

Turn initiated at 360 degrees to the left

Initiate rollout at (45 / 2) 22.5 degrees prior to reaching 360, i.e. 023 degrees.

Calculating VDP (or PDP, Planned Descent Point)

Distance-based VDP/PDP:

MDA (HAT) / 300

Example:

MDA is 430 ft. AGL.  430/300 = 1.4nm

Time-based VDP/PDP:

Subtract 10% of the MDA (HAT) from the time between FAF and MAP

Example:

MDA is 430 ft. AGL

Time between FAF and MAP is 3:54

10% of 430 = 43

Subtract 43 seconds from 3:54 = 3:11 to VDP/PDP