PERFORMANCE SUMMARY

PIPER TWIN COMANCHE PA-30B SERIAL #30-1386

# Normally aspirated – no tip tanks – 160hp per side

V-speeds (mph)

| VA | Design Maneuvering Speed and Turbulent Air Penetration Speed | 162 |
| --- | --- | --- |
| VAPP | Final Approach to Landing Speed | 95 |
| VFE | Flap Extension Speed | 125 |
| VFE | Flap Extension Speed, recommended | 100 |
| VLO | Landing Gear Operation Speed | 150 |
| VLO | Landing Gear Operation Speed, recommended | 125 |
| VLE | Landing Gear Extended Speed | 150 |
| VLE | Landing Gear Extended Speed, recommended | 125 |
| VMCA | Single Engine Minimum Control Speed | 90 |
| VNE | Never Exceed Speed | 230 |
| VR | Rotation Speed | 90 |
| VSO | Stall Speed, Full Flaps and Gear Extended | 69 |
| VS1 | Stall Speed, Flaps and Gear Retracted | 76 |
| VSSE | Minimum Intentional Single Engine Speed | 97 |
| VX | Best Angle-of-Climb Speed, sea level | 90 |
| VXSE | Best Single Engine Angle-of-Climb Speed | 94 |
| VY | Best Rate-of-Climb Speed, sea level | 112 |
| VYSE | Best Single Engine Rate-of-Climb Speed | 105 |
|  | Best En Route Rate-of-Climb Speed | 130 |
|  | Demonstrated Crosswind Component | 20 |

Absolute ceiling: 20,000 feet density altitude

Single-engine absolute ceiling: 7,100 feet density altitude

Service ceiling: 18,600 feet density altitude

Single-engine service ceiling: 5,800 feet density altitude

Maximum approved altitude: FL220

- based on MGTOW of 3600 lb.

Demonstrated crosswind component: 20mph

Rate of climb (sea level, ISA, 3600 lbs.): 1,460 FPM

Climb gradient (sea level, ISA, 3600 lbs.): 903 FPNM

Takeoff ground run distance (80 F, sea level, 3400 lbs., 10 knots wind): 1100 feet

Takeoff ground run distance (same conditions, no wind): 1300 feet

50 ft. obstacle takeoff ground run distance (80 F, sea level, 3400 lbs., 10 knots wind): 2200 feet

50 ft. obstacle takeoff ground run distance (same conditions, no wind): 1900 feet

Accelerate-stop (p. 5-12 of PA-30B AFM)

3600 lb., sea level, ISA, no wind: @ 3000 feet runway distance.

Best power EGT: 100 degrees F ROP at 75% power

Best economy EGT: Peak EGT (recommended <75% power)

Simulated feather power settings (from “PA-30 and Multi-Engine Flying” by Alice S. Fuchs, p. 34)

Props and mixtures: full forward

Throttle: slightly less than 10” MAP

Adjust throttle slightly for the following speeds/tach settings:

100 mph	1980 RPM

110 mph	2180 RPM

120 mph	2380 RPM

130 mph	2580 RPM